Base Station Assembly Guide

Rev 1.0 – RG – 22nd May 2015

Parts

C

A

B

E

D

A – 4x Screws 4x1 Black thread forming (BOM item 30) – Customer supplied.

B – 4x Spacers M3 NYLON 3MM (BOM item 31).

C – 1x Assembled PCB.

D – 1x Enclosure base (BOM item 29) – Customer supplied.

E – 1x Enclosure lid (BOM item 29) – Customer supplied.

Step 1 – With enclosure lid E face down, place assembled PCB C into enclosure lid E.

Step 2 – Insert screws A into enclosure lid E and through assembled PCB C.

Step 3 – Lay back face down and place spacers B on to screws A.

Step 3 – Place enclosure base D face down, on to the screws protruding enclosure lid E.

Step 3 (continued)

Step 4 – Carefully flip entire assembly over, so the enclosure base D is on the bottom.

Step 5 – Screw in all four screws A until tight. Enclosure manufacturer recommends torque of 4 lbs.in.

Assembly complete.